Excellent. These decisions remove most of the architectural ambiguity and give you a much cleaner production-oriented system design.

You now have a coherent heterogeneous edge AI architecture with:

- deterministic control separation

- proper workload specialization

- edge-first resilience

- production-oriented performance constraints

- hardware-realistic communication paths

Below is the updated consolidated architecture specification after applying all chosen resolutions.

# Finalized System Architecture

Camera
   │
   ▼
Jetson Nano
────────────────────────────
AI Perception Engine
────────────────────────────
- YOLOv8 TensorRT inference
- CUDA preprocessing
- OpenCV CUDA pipeline
- ByteTrack tracking
- Vehicle counting
- Trajectory extraction
- Metadata generation
- GPU acceleration
   │
   │ metadata/events
   │ UART
   ▼
STM32 Nucleo N657X0-Q
────────────────────────────
Deterministic Analytics Engine
────────────────────────────
- Queue estimation
- Temporal filtering
- Traffic-state classification
- Adaptive inference scheduling
- Fixed-point DSP analytics
- Real-time control logic
- Scheduler authority
   │
   │ telemetry
   │ UART/CAN
   ▼
Raspberry Pi 5
────────────────────────────
Edge Gateway
────────────────────────────
- Mosquitto MQTT broker
- Edge orchestration
- Local buffering/cache
- Edge API
- Health monitoring
- Temporary storage
- Cloud bridge
   │
   │ MQTT / HTTPS
   ▼
Cloud Backend
────────────────────────────
Central Analytics Platform
────────────────────────────
- PostgreSQL
- Long-term analytics
- Grafana dashboards
- Fleet management
- Historical storage
- Global optimization

# Finalized Responsibility Ownership

This is now clearly separated.

# Jetson Nano Responsibilities

## Owns ALL perception tasks

### Responsibilities

- Camera acquisition

- CUDA preprocessing

- TensorRT inference

- YOLOv8 detection

- ByteTrack tracking

- Vehicle counting

- Trajectory extraction

- Speed estimation

- Metadata/event generation

## Jetson Output

Jetson produces metadata only.

Example:

{
  "track_id": 42,
  "class": "car",
  "center_x": 551,
  "center_y": 318,
  "velocity": 12.4,
  "direction": "northbound",
  "crossed_line": true,
  "timestamp": 1715502000
}

# STM32 Responsibilities

## Owns deterministic analytics and control

STM32 does NOT process:

- video

- frames

- YOLO inference

STM32 consumes metadata only.

## Responsibilities

### Real-Time Analytics

- queue estimation

- temporal smoothing

- congestion scoring

- flow-rate estimation

- lane occupancy estimation

### Real-Time Control

- adaptive inference scheduling

- traffic-state transitions

- deterministic timing analysis

- RTOS scheduling

- future traffic-light control

### DSP Acceleration

- CMSIS-DSP fixed-point filtering

- moving averages

- signal smoothing

- predictive estimation

# Final Adaptive Inference Scheduling Design

This is now cleanly defined.

# STM32 = Scheduling Authority

STM32 monitors:

- traffic density

- queue lengths

- congestion states

- event frequency

Then dynamically adjusts Jetson workload.

# Example Commands

{
  "inference_interval_ms": 30,
  "tracking_mode": "HIGH_ACCURACY",
  "frame_skip": 0
}

Low traffic:

{
  "inference_interval_ms": 200,
  "tracking_mode": "LOW_POWER",
  "frame_skip": 4
}

# Key Design Principle

Jetson performs perception.

STM32 controls computational policy.

This is a strong heterogeneous computing design.

# Final Raspberry Pi Role

The Raspberry Pi is now strictly an edge gateway.

# Raspberry Pi Responsibilities

## Edge Infrastructure Layer

### MQTT

- Mosquitto broker hosting

- topic routing

- local pub/sub

### Edge Orchestration

- service supervision

- device heartbeat

- health monitoring

### Local Resilience

- buffering

- temporary cache

- retry queues

### API Layer

- edge REST API

- local diagnostics

### Cloud Bridge

- secure upload

- synchronization

- offline recovery

# Raspberry Pi DOES NOT Own

- primary AI inference

- deterministic scheduling

- heavy analytics

- long-term analytics

# Final Cloud Responsibilities

Cloud becomes centralized analytics infrastructure.

# Cloud Responsibilities

## Data Platform

- PostgreSQL primary storage

- historical analytics

- traffic trends

- reporting

## Fleet Management

- node monitoring

- deployment orchestration

- OTA management (future)

## Visualization

- Grafana dashboards

- city-wide analytics

## Future Extensions

- prediction models

- anomaly detection

- federated learning

- model management

# Final MQTT Architecture

## MQTT Broker Placement

### Final Decision

Mosquitto runs on Raspberry Pi.

# Why This Is Correct

Advantages:

- edge autonomy

- low latency

- offline capable

- scalable

- local resilience

If cloud disconnects:

- local system still functions

This is critical for smart-city infrastructure.

# Final Communication Stack

| Connection | Protocol |
| --- | --- |
| Camera → Jetson | CSI |
| Jetson → STM32 | UART |
| STM32 → Pi | UART initially |
| Pi → Cloud | MQTT + HTTPS |
| Pi internal messaging | localhost MQTT |

# Future Upgrade Path

Later:

- CAN FD

- Ethernet

- TSN

can replace UART if scaling increases.

UART is correct for MVP.

# Final ByteTrack Language Boundary

## Final Decision

### Production Path

C++ first.

### Preferred Stack

- OpenCV C++

- CUDA C++

- TensorRT C++

- ByteTrack C++

Python becomes:

- prototyping only

- experiments only

- tooling only

# Why This Matters

This aligns with:

- zero-copy pipelines

- lower latency

- better threading

- native TensorRT APIs

- deterministic memory control

This is the correct production-grade decision.

# Final Zero-Copy Strategy

This is now a hard architectural constraint.

# Hard Rule

Avoid:

GPU → CPU → GPU

Whenever possible.

# Final GPU Pipeline

Camera Frame
   ↓
CUDA upload
   ↓
cv::cuda::GpuMat
   ↓
CUDA preprocessing
   ↓
TensorRT inference
   ↓
GPU postprocessing
   ↓
minimal CPU metadata extraction
   ↓
tracking/counting

# Memory Strategy

## Required Techniques

### Use

- CUDA pinned memory

- GPU memory reuse

- unified memory carefully

- GpuMat pipelines

- preallocated buffers

### Avoid

- repeated malloc/free

- unnecessary frame copies

- CPU intermediate buffers

# Final Simulation Philosophy

This is now extremely important:

Simulation code must be deployment-compatible.

Meaning:

- same interfaces

- same schemas

- same MQTT topics

- same metadata structures

- same APIs

- same threading model

# Correct Simulation Strategy

## GOOD

SUMO
  ↓
Synthetic frames
  ↓
REAL production inference pipeline
  ↓
REAL MQTT topics
  ↓
REAL FastAPI backend

## BAD

Fake prototype-only code

that later gets rewritten entirely.

# Final Development Philosophy

## Prototype the REAL architecture early

Simulation should emulate:

- deployment interfaces

- timing behavior

- communication patterns

- metadata formats

This massively reduces integration problems later.

# Final Production-Oriented Monorepo

smart-traffic-system/
│
├── jetson-ai/
│   ├── src/
│   ├── include/
│   ├── detection/
│   ├── tracking/
│   ├── analytics/
│   ├── cuda/
│   ├── mqtt/
│   ├── models/
│   ├── cmake/
│   └── CMakeLists.txt
│
├── stm32-realtime/
│   ├── Core/
│   ├── Drivers/
│   ├── DSP/
│   ├── FreeRTOS/
│   ├── traffic_engine/
│   └── communication/
│
├── raspberrypi-gateway/
│   ├── mqtt/
│   ├── edge_api/
│   ├── orchestration/
│   ├── buffering/
│   └── monitoring/
│
├── cloud/
│   ├── backend/
│   ├── storage/
│   ├── dashboards/
│   ├── analytics/
│   └── deployment/
│
├── shared/
│   ├── protobuf/
│   ├── schemas/
│   ├── configs/
│   └── protocol/
│
├── simulation/
│   ├── sumo/
│   ├── synthetic_camera/
│   └── freertos_sim/
│
└── docs/

# Remaining Major Decisions

You have resolved most major architectural contradictions.

The next important unresolved decisions are now:

# Remaining Open Architecture Decisions

## 1. Serialization Format

Choose:

- JSON

- Protobuf

- FlatBuffers

- Cap’n Proto

Recommendation:

- JSON for MVP

- Protobuf later

## 2. Threading Framework

Choose:

- std::thread

- TBB

- OpenMP

- custom thread pools

Recommendation:

- std::thread initially

## 3. Build System Standardization

Choose:

- pure CMake

- Bazel

- Meson

Recommendation:

- modern CMake

## 4. Logging Infrastructure

Choose:

- spdlog

- glog

- custom logger

Recommendation:

- spdlog

## 5. Configuration Management

Choose:

- YAML

- TOML

- JSON

Recommendation:

- YAML configs

# Most Important Next Step

You are now ready for:

# Interface Control Documents (ICDs)

This should be your immediate next specification phase.

Especially:

- UART packet format

- MQTT schema

- Traffic metadata schema

- scheduler command schema

- state machine definitions

- threading model

- timing guarantees

- failure/recovery behavior

That will convert this from “architecture ideas” into an implementable engineering specification.