# Smart Traffic Hybrid Edge AI System — Template Project

## Project Goal

Build a distributed real-time traffic analytics system using:

- NVIDIA Jetson Nano for AI inference

- STM32 Nucleo N for deterministic real-time analytics

- Raspberry Pi for orchestration/networking/cloud communication

The system focuses on:

- extremely accurate vehicle counting

- real-time traffic analytics

- adaptive AI inference pipelines

- distributed edge computing

- heterogeneous processing

# Final Architecture

CAMERA
   │
   ▼
STM32 NUCLEO N
(Real-time analytics engine)
   │
   ▼
JETSON NANO
(YOLO detection + ByteTrack)
   │
   ▼
RASPBERRY PI
(MQTT + API + Database + Dashboard)
   │
   ▼
CLOUD

# Performance-First Engineering Philosophy

This project prioritizes:

- maximum real-time performance

- low latency

- deterministic execution

- GPU acceleration

- native memory management

- multithreading efficiency

- zero-copy processing when possible

Therefore:

# C++ is the PRIMARY language.

Python is used only for:

- rapid prototyping

- testing

- AI experimentation

- model conversion utilities

The production system should be predominantly implemented in:

- C++

- CUDA

- OpenCV C++

- TensorRT

- FreeRTOS C

# Recommended Software Stack

| Layer | Technology |
| --- | --- |
| Core Language | C++ |
| GPU Compute | CUDA |
| AI Detection | YOLOv8 |
| Tracking | ByteTrack |
| AI Acceleration | TensorRT |
| Vision | OpenCV |
| Messaging | MQTT |
| Broker | Mosquitto |
| Backend API | FastAPI |
| Database | PostgreSQL |
| Dashboard | Grafana |
| STM32 RTOS | FreeRTOS |
| STM32 DSP | CMSIS-DSP |
| STM32 AI | X-CUBE-AI |

# Monorepo Structure

smart-traffic-system/
│
├── jetson-ai/
│   ├── detection/
│   ├── tracking/
│   ├── analytics/
│   ├── mqtt/
│   ├── config/
│   ├── models/
│   └── main.py
│
├── stm32-realtime/
│   ├── Core/
│   ├── Drivers/
│   ├── Middlewares/
│   ├── DSP/
│   ├── FreeRTOS/
│   └── traffic_engine/
│
├── raspberrypi-gateway/
│   ├── api/
│   ├── mqtt/
│   ├── database/
│   ├── dashboard/
│   ├── analytics/
│   └── main.py
│
├── cloud/
│   ├── backend/
│   ├── storage/
│   ├── dashboards/
│   └── deployment/
│
├── shared/
│   ├── protobuf/
│   ├── schemas/
│   └── configs/
│
└── docs/

# C++-First System Design

## Why C++

The traffic system is real-time and performance-critical.

C++ provides:

- deterministic memory management

- lower latency

- direct CUDA integration

- better multithreading

- lower overhead

- native TensorRT support

- superior OpenCV performance

- easier SIMD optimization

Python introduces:

- GIL limitations

- higher latency

- memory overhead

- reduced determinism

Therefore:

| Component | Recommended Language |
| --- | --- |
| Jetson inference engine | C++ |
| CUDA kernels | CUDA C++ |
| OpenCV processing | C++ |
| tracking engine | C++ |
| STM32 firmware | C |
| Raspberry Pi backend | C++ or mixed |
| dashboards | Python/web |

# 1. Jetson Nano AI Module

## Responsibilities

- vehicle detection

- multi-object tracking

- counting

- inference optimization

- event generation

# Jetson Dependencies

## Jetson C++ Dependencies

sudo apt update
sudo apt install python3-pip

sudo apt install build-essential cmake git
sudo apt install libopencv-dev
sudo apt install nvidia-cuda-toolkit

# TensorRT
sudo apt install tensorrt

# MQTT
sudo apt install libmosquitto-dev

# JSON
sudo apt install nlohmann-json3-dev

---

# Jetson Main Pipeline

## File

```text
jetson-ai/src/main.cpp

## High-Performance C++ Template

#include <opencv2/opencv.hpp>
#include <mosquitto.h>
#include <iostream>
#include <vector>

int main()
{
    cv::VideoCapture cap(0);

    if (!cap.isOpened())
    {
        std::cerr << "Camera failed" << std::endl;
        return -1;
    }

    cv::Mat frame;

    while (true)
    {
        cap >> frame;

        if (frame.empty())
            break;

        // CUDA preprocessing
        // TensorRT inference
        // ByteTrack update
        // Counting logic

        cv::imshow("Traffic", frame);

        if (cv::waitKey(1) == 27)
            break;
    }

    return 0;
}

# ByteTrack Integration

## Recommended Structure

jetson-ai/tracking/
├── bytetrack_wrapper.py
├── counter.py
└── trajectory.py

# Counting Logic

## Virtual Counting Line

COUNT_LINE_Y = 400

if center_y > COUNT_LINE_Y:
    total_count += 1

# CUDA + TensorRT Pipeline

## Recommended GPU Pipeline

Camera
   ↓
CUDA preprocessing
   ↓
TensorRT inference
   ↓
GPU postprocessing
   ↓
ByteTrack tracking
   ↓
Counting engine

# OpenCV CUDA Acceleration

## Recommended Modules

| OpenCV CUDA Module | Purpose |
| --- | --- |
| cv::cuda::GpuMat | GPU memory |
| cv::cuda::resize | GPU resize |
| cv::cuda::cvtColor | GPU colorspace conversion |
| cv::cuda::GaussianBlur | GPU filtering |
| cv::cuda::threshold | GPU thresholding |

# Example CUDA OpenCV Usage

cv::Mat frame;
cv::cuda::GpuMat gpu_frame;

gpu_frame.upload(frame);

cv::cuda::GpuMat resized;

cv::cuda::resize(
    gpu_frame,
    resized,
    cv::Size(640, 640)
);

# Recommended Build System

## CMake

cmake_minimum_required(VERSION 3.10)
project(SmartTrafficSystem)

find_package(OpenCV REQUIRED)
find_package(CUDA REQUIRED)

add_executable(traffic
    src/main.cpp
)

target_link_libraries(traffic
    ${OpenCV_LIBS}
)

# TensorRT Optimization

## Export Model

yolo export model=yolov8s.pt format=engine

# STM32 Real-Time Analytics Engine

## Responsibilities

The STM32 does NOT run YOLO.

Instead it handles:

- deterministic analytics

- tracking smoothing

- queue estimation

- timing analysis

- adaptive inference scheduling

- traffic state classification

- fixed-point DSP acceleration

# STM32 Architecture

traffic_engine/
├── scheduler.c
├── queue_estimator.c
├── temporal_filter.c
├── motion_predictor.c
├── state_classifier.c
└── analytics_pipeline.c

# FreeRTOS Task Layout

| Task | Priority |
| --- | --- |
| Frame Scheduler | High |
| Tracking Filter | High |
| Queue Estimator | Medium |
| MQTT Serialization | Medium |
| Diagnostics | Low |

# Example FreeRTOS Task

void QueueEstimatorTask(void *argument)
{
    while (1)
    {
        compute_queue_density();
        compute_flow_rate();

        vTaskDelay(pdMS_TO_TICKS(10));
    }
}

# CMSIS-DSP Usage

## Include

#include "arm_math.h"

## Example

arm_mean_f32(input_buffer, BUFFER_SIZE, &mean_value);

# Traffic State Classifier

## States

EMPTY
LIGHT
MODERATE
HEAVY
CONGESTED
BLOCKED

# Adaptive Inference Scheduler

## Concept

STM32 dynamically controls:

- inference rate

- processing bursts

- frame skipping

- queue priorities

# Example Logic

if (traffic_density < LOW_THRESHOLD)
{
    inference_interval_ms = 200;
}
else
{
    inference_interval_ms = 30;
}

# Raspberry Pi Gateway

The Raspberry Pi should also prioritize C++ services for:

- MQTT processing

- event routing

- stream aggregation

- low-latency APIs

Python should mainly be used for:

- dashboards

- quick scripts

- experiments

- visualization

# High-Performance Design Recommendations

## Use Multithreading

Recommended threads:

| Thread | Purpose |
| --- | --- |
| Capture thread | camera input |
| GPU inference thread | TensorRT |
| Tracking thread | ByteTrack |
| MQTT thread | networking |
| Analytics thread | statistics |

# Use Lock-Free Queues

Avoid blocking pipelines.

Recommended:

- ring buffers

- producer-consumer queues

- atomic counters

# Prefer Zero-Copy Transfers

Minimize CPU↔GPU copies.

Use:

- CUDA pinned memory

- unified memory

- GpuMat pipelines

# Use Fixed-Point Math on STM32

Avoid floating point when unnecessary.

Use:

- CMSIS-DSP

- SIMD instructions

- integer arithmetic

# Recommended Optimization Priorities

| Optimization | Importance |
| --- | --- |
| TensorRT | Critical |
| CUDA preprocessing | Critical |
| Multithreading | Critical |
| GPU memory reuse | High |
| Frame skipping | High |
| Batch processing | Medium |
| Quantization | Medium |

# Recommended Performance Targets

| Metric | Target |
| --- | --- |
| Detection FPS | 20–30 FPS |
| Counting latency | <100ms |
| MQTT latency | <20ms |
| STM32 task jitter | <1ms |
| GPU utilization | >70% efficient |

# Raspberry Pi Gateway

## Responsibilities

- MQTT broker/client

- orchestration

- database storage

- dashboard hosting

- API layer

- cloud communication

# Install Dependencies

pip install fastapi
pip install uvicorn
pip install psycopg2-binary
pip install paho-mqtt

# MQTT Topic Design

| Topic | Purpose |
| --- | --- |
| traffic/raw | raw AI metadata |
| traffic/counts | counting events |
| traffic/analytics | processed statistics |
| traffic/state | congestion state |
| traffic/system | diagnostics |

# FastAPI Backend

## File

raspberrypi-gateway/api/main.py

## Template

from fastapi import FastAPI

app = FastAPI()

@app.get("/traffic/status")
def status():
    return {
        "system": "online"
    }

# PostgreSQL Schema

## Vehicle Counts

CREATE TABLE vehicle_counts (
    id SERIAL PRIMARY KEY,
    timestamp TIMESTAMP,
    intersection_id VARCHAR(32),
    vehicle_count INTEGER,
    congestion_level FLOAT
);

# Grafana Dashboard Metrics

## Suggested Panels

- vehicles/minute

- congestion level

- lane occupancy

- queue length

- peak traffic hours

- inference FPS

- Jetson GPU utilization

- STM32 task timing

# MQTT Example

## Raspberry Pi Subscriber

import paho.mqtt.client as mqtt


def on_message(client, userdata, msg):
    print(msg.topic)
    print(msg.payload.decode())


client = mqtt.Client()
client.on_message = on_message

client.connect("localhost", 1883)
client.subscribe("traffic/#")
client.loop_forever()

# Recommended Communication Design

| Connection | Protocol |
| --- | --- |
| Jetson → Pi | MQTT |
| Pi → Cloud | HTTPS/MQTT |
| STM32 → Pi | UART/CAN |
| STM32 → Jetson | UART/SPI |

# Recommended Development Roadmap

## Phase 1

### Basic AI Detection

- camera feed

- YOLO inference

- object counting

## Phase 2

### Tracking + Analytics

- ByteTrack

- counting lines

- congestion estimation

## Phase 3

### STM32 Integration

- FreeRTOS

- DSP analytics

- adaptive inference scheduling

## Phase 4

### Distributed Networking

- MQTT

- PostgreSQL

- FastAPI

- Grafana

## Phase 5

### Optimization

- TensorRT

- fixed-point acceleration

- temporal filtering

- TinyML traffic-state models

# Most Important Design Principle

Do NOT overload one processor.

Instead:

| Processor | Responsibility |
| --- | --- |
| Jetson | perception |
| STM32 | deterministic analytics |
| Raspberry Pi | orchestration |

This is the correct heterogeneous edge AI architecture.

# Final Project Definition

## Project Title

Hybrid Edge AI Traffic Analytics System using NVIDIA Jetson Nano, STM32 Nucleo N, and Raspberry Pi

## Key Technologies

- Edge AI

- Embedded DSP

- Real-Time Systems

- Distributed Computing

- Heterogeneous Processing

- GPU Acceleration

- TinyML

- MQTT IoT Networking

# Recommended Future Extensions

- multi-camera fusion

- traffic prediction

- anomaly detection

- edge clustering

- federated learning

- AI model hot-swapping

- adaptive distributed inference

I updated the template project to make it fully performance-oriented with a C++-first architecture, including:

- CUDA-focused GPU pipeline design

- OpenCV C++ acceleration

- TensorRT integration

- multithreaded architecture

- lock-free queue recommendations

- zero-copy GPU transfer strategy

- CMake build system

- STM32 fixed-point DSP optimization

- production-grade performance targets

- reduced reliance on Python

The project now reflects a real high-performance heterogeneous edge computing system rather than a prototype AI demo.

Simulated version

Yes — and for your architecture, simulation is actually one of the best ways to prototype early without needing all the hardware immediately.

For your project, you can realistically simulate:

- traffic flow

- camera streams

- vehicle counting

- STM32 logic

- distributed communication

- cloud pipeline

- MQTT traffic

- real-time analytics

before deploying to real hardware.

# Best Simulation Strategy

You should split simulation into layers.

| Layer | Simulate? | Best Tool |
| --- | --- | --- |
| Traffic movement | Yes | SUMO |
| Camera/video | Yes | prerecorded video / CARLA |
| Jetson AI pipeline | Yes | PC/Linux GPU |
| MQTT networking | Yes | Mosquitto |
| Raspberry Pi backend | Yes | Docker/Linux |
| STM32 logic | Partially | STM32CubeIDE simulator |
| Real-time scheduling | Partially | FreeRTOS simulation |

# BEST OVERALL TOOL

# Simulation of Urban MObility (SUMO)

This is probably the best simulator for your project.

# Why SUMO Fits PERFECTLY

SUMO can simulate:

- realistic traffic

- intersections

- lanes

- congestion

- traffic density

- vehicle flow

- routing

- vehicle spawning

- timing

Then your AI system can consume this data.

# Recommended Prototype Architecture

SUMO Traffic Simulation
        │
        ▼
Synthetic Camera Feed
        │
        ▼
Jetson AI Pipeline (on PC)
        │
        ▼
MQTT
        │
        ▼
Raspberry Pi Backend

This lets you prototype:

- counting logic

- analytics

- networking

- dashboards

without physical hardware.

# Option 1 — SUMO + OpenCV

This is the BEST starting point.

## Workflow

SUMO generates:

- moving vehicles

- lanes

- traffic states

You:

- capture frames

- feed them into YOLO

- run counting/tracking

# Option 2 — CARLA

More advanced.

CARLA gives:

- realistic 3D environments

- weather

- camera sensors

- semantic segmentation

- traffic physics

This is MUCH heavier computationally but amazing for research.

# Recommended Usage

Use CARLA if you want:

- photorealistic traffic

- realistic camera perspectives

- advanced AI research

Use SUMO if you want:

- scalable traffic analytics

- fast prototyping

- traffic engineering focus

# Best Practical Combination

Honestly:

# SUMO + OpenCV + YOLO

is the sweet spot for your project.

# Simulating the Jetson Nano

You do NOT initially need the actual:

NVIDIA Jetson Nano

You can prototype on:

- desktop Linux

- WSL2

- Ubuntu PC

- gaming GPU

using:

- CUDA

- TensorRT

- OpenCV

Then deploy later.

# Simulating STM32 Logic

This is trickier but still possible.

# Option A — STM32CubeIDE

STM32CubeIDE

Supports:

- debugging

- peripheral simulation

- FreeRTOS development

Good for:

- firmware structure

- scheduling

- task timing

# Option B — FreeRTOS POSIX/Linux Simulation

This is VERY useful.

You can run FreeRTOS tasks on Linux.

Meaning:

- your STM32 architecture can be prototyped on PC first.

This is actually excellent for:

- queue estimators

- analytics engine

- temporal filtering

- scheduler logic

# Example

Your STM32 tasks become:

QueueEstimatorTask
TrackingFilterTask
InferenceSchedulerTask

running on:

- Linux threads first

- STM32 later

This is professional workflow.

# Simulating MQTT + Cloud

Very easy.

Use:

- Mosquitto

- Docker

- FastAPI

- PostgreSQL

- Grafana

all locally.

# VERY POWERFUL PROTOTYPE SETUP

You can simulate the ENTIRE SYSTEM on one PC.

# Full Virtual Prototype

SUMO
  ↓
Synthetic video
  ↓
YOLO + ByteTrack
  ↓
MQTT
  ↓
FastAPI
  ↓
PostgreSQL
  ↓
Grafana Dashboard

This is already a serious prototype.

# Most Valuable Part to Simulate

The MOST important thing to prototype early is:

# tracking + counting accuracy

NOT hardware deployment.

# Recommended Prototype Development Order

# Phase 1 — Traffic Simulation

Use:

- SUMO

Goal:

- generate traffic patterns

# Phase 2 — AI Counting

Use:

- YOLOv8

- ByteTrack

- OpenCV C++

Goal:

- accurate counting

# Phase 3 — Distributed Messaging

Add:

- MQTT

- FastAPI

Goal:

- real-time analytics pipeline

# Phase 4 — STM32 Logic Simulation

Use:

- FreeRTOS simulation

Goal:

- deterministic analytics engine

# Phase 5 — Hardware Deployment

Finally deploy:

- Jetson Nano

- Raspberry Pi

- STM32 Nucleo N

# Best Academic/Engineering Advantage

Simulation lets you test:

- congestion

- edge cases

- high traffic density

- night conditions

- scaling

- multi-lane scenarios

much faster than real-world testing.

# Best Tools Summary

| Purpose | Tool |
| --- | --- |
| Traffic simulation | SUMO |
| Realistic traffic rendering | CARLA |
| AI inference | OpenCV + TensorRT |
| Tracking | ByteTrack |
| STM32 prototyping | FreeRTOS simulator |
| MQTT | Mosquitto |
| Dashboard | Grafana |
| Backend | FastAPI |
| Database | PostgreSQL |

# My Strong Recommendation

Start with:

# SUMO + OpenCV C++ + YOLOv8 + ByteTrack

on your PC first.

Then:

- add MQTT

- add dashboards

- simulate STM32 tasks

- optimize performance

- finally move to embedded hardware

That will massively accelerate development and reduce debugging pain.